Endi to'liq zanjirni, aniq fayl-fayl, va har bir formulani kod ichidan tirnoqqa olib chiqaraman — hech narsa o'ylab topmayman, faqat skriptdan ko'chiraman.# To'liq pipeline arxitekturasi — run_sebal.pydan boshlab, har bir formula skriptdan olingan

## 1-QISM: Chaqiruv zanjiri (run_sebal.py → oxirigacha)

run_sebal.py

│

├─ ee.Initialize()

├─ ee_utils.install_getinfo_retry()   # 429 xatoga qarshi

│

└─ main.run(roi_type='gaul', mode='pysebal', satellite='HLS', ...)

│

└─ [har tile uchun] process_tile(roi, date_start, date_end, mode, satellite, cloud_max)

│

├─ 1) preprocessing.build_collection()

│      → apply_qa_mask() / apply_qa_mask_hls()

│      → apply_scale_factors() / apply_scale_factors_hls()

│      → add_terrain() [DEM, slope]

│      → get_era5_for_image() [wind, temp, pressure, ssrd, strd, dewpoint]

│      → add_air_density()

│

├─ 2) collection.map(surface_props.compute_all)

│      → compute_ndvi → compute_savi → compute_albedo

│      → compute_emissivity → compute_z0m

│      → compute_transmissivity → compute_lai

│

├─ 3) collection.map(radiation.compute_all)

│      → compute_incoming_shortwave (K↓)

│      → compute_incoming_longwave (L↓)

│      → compute_outgoing_longwave (L↑)

│      → compute_net_radiation (Rn)

│      → compute_soil_heat_flux (G0)

│      → compute_net_available_energy (RN_G0)

│

├─ 4) [har sahna, alohida] energy_balance.compute_all(img, roi)

│      → select_anchor_pixels (cold/hot)

│      → compute_friction_velocity (u*)

│      → compute_rah_neutral (rah)

│      → compute_sensible_heat_flux (H, Monin-Obukhov iteratsiya)

│      → compute_latent_heat_flux (λE)

│      → compute_etrf / compute_evaporative_fraction (ETrF, EVAP_FRAC)

│

├─ 5) daily_et.compute_daily_et(img, roi)

│      → get_daily_solar_radiation (Rs24, ERA5)

│      → Rn24, ET_24

│

└─ 6) [FAQAT mode='pysebal' bo'lsa]

et_decomposition.compute_all(img)

→ compute_vapor_pressure, compute_etref, compute_etpot,

compute_advection_factor, compute_crop_coefficients,

compute_et_separation

→ soil_moisture.compute_all(img)   [SMAP]

→ biomass.compute_all(img)         [FPAR→APAR→LUE→Biomass]

→ irrigation.compute_all(img)       [sug'orish klassi]

Keyin main.run() ichida:

├─ _export_daily()      [agar export_daily=True]

├─ _export_monthly()    → mode='pysebal': monthly_analytics.compute_all_monthly()

│                          mode='maqola':  daily_et.compute_monthly_et()

├─ _viirs_export_month() [agar use_viirs=True]

├─ _s30_export_month()   [agar use_s30_etrf=True]

└─ validation.validate() [agar validate=True]

## 2-QISM: Har modulning aniq formulasi va konstantasi (skriptdan olingan, o'zgartirilmagan)

### preprocessing.py

SR = DN × 0.0000275 - 0.2                      (clamp 0-1)

LST = DN × 0.00341802 + 149.0                  [Kelvin]  (Landsat)

LST_HLS = B10 + 273.15                          [Kelvin]  (HLS — Celsius kirish)

Wind_speed = sqrt(u_wind² + v_wind²)

ρₐ = P / (287.058 × T)                          [R_specific = 287.058 J/(kg·K)]

ERA5 oynasi: overpass ±1 soat, .mean()

### surface_props.py

NDVI = (NIR-Red)/(NIR+Red)

SAVI = ((NIR-Red)/(NIR+Red+0.5)) × 1.5           [L=0.5, Huete 1988]

Albedo = 0.246×B2+0.146×B3+0.191×B4+0.304×B5+0.105×B6+0.008×B7   [Olmedo 2016], clamp(0,0.70)

ε₀ = 1.009 + 0.047×ln(NDVI)                      [0.16≤NDVI≤0.74]

NDVI<0 → 0.985 (suv) | 0≤NDVI<0.16 → 0.960 | NDVI>0.74 → 0.985

z₀m = exp(-5.809 + 5.62×SAVI)                    clamp(0.0002, 1.0)

z₀h = z₀m / exp(2.3)                              [kB⁻¹=2.3]

τsw = 0.75 + 2×10⁻⁵ × elevation                   [Allen 2007]

LAI = -ln((0.69-SAVI)/0.59)/0.91                  [0.1≤SAVI<0.687], SAVI≥0.687→6.0

### radiation.py

K↓ = (ERA5_SSRD/3600) × (τsw/0.75)

L↓ = ERA5_STRD/3600

L↑ = ε₀ × σ × T₀⁴                                  [σ=5.67×10⁻⁸]

Rn = (1-α)×K↓ + L↓ - L↑ - (1-ε₀)×L↓

G0 = Rn × (T₀-273.15)/α × (0.0038α+0.0074α²) × (1-0.978×NDVI⁴)

Suv/NDVI<0 → G0 = 0.5×Rn

clamp: G0 ∈ [-0.10×Rn, 0.50×Rn]

RN_G0 = Rn - G0

Rn24 = (1-α)×Rs24 - 110×τsw   [110 = De Bruin 1987]  ← compute_rn24, ISHLATILMAYDI

### energy_balance.py

u_200 = u_10 × ln(200/0.12) / ln(10/0.12)

u* = k × u_200 / ln(200/z0m)   [k=0.41]                clamp min 0.02

rah_neutral = ln(2.0/0.1) / (k×u*)                      clamp min 1.0

δTa kalibratsiya:

dTa_hot = H_hot × rah_hot / (ρ×cp)      [cp=1004]

dTa_cold = 0

c4 = dTa_hot/(T_hot-T_cold),  c5 = -c4×T_cold

dTa(x,y) = c4×T₀(x,y) + c5

H = ρ×cp×dTa / rah

L_MO = -ρ×cp×u*³×T₀ / (k×g×H)                            [g=9.81]

ψm, ψh — Paulson (1970), unstable: x=(1-16z/L)^0.25; stable: ψ=-5z/L

λE = Rn - G0 - H

ETrF = λE/(Rn-G0), clamp(0,1.5)

EVAP_FRAC (Λ) = λE/(Rn-G0), clamp(0,1.0)   ← ETrF bilan bir xil formula

### daily_et.py

Rs24 = ERA5_SSRD.sum(24h) / 86400

Rn24 = (1-α)×Rs24 - 110×τsw

ET_24 = Λ × Rn24 × (86400/2.45×10⁶)                     [λ=2.45×10⁶ J/kg]

Oylik: Λ interpolyatsiya (chiziqli, before/after Landsat sahna orasida)

→ har kun ET_kun → Σ = ET_MONTHLY

### et_decomposition.py

esat = 0.6108×exp(17.27×T/(T+237.3))

eact = 0.6108×exp(17.27×Td/(Td+237.3))

slope_es = 4098×esat/(T+237.3)²

psychro γ = P(kPa)×0.000665

u₂ = u₁₀×0.745                                          [≈4.87/ln(67.8×10-5.42)]

ETref = [0.408×Δ×Rn24 + γ×900/(Ta+273)×u₂×VPD] / [Δ+γ×(1+0.34×u₂)]

rs_min = 100/max(LAI,0.5), rah_pot = max(208/u₂, 25)     [ETpot uchun]

AF (advection) = 1 + 0.985×[exp(VPD×0.08)-1]×EF

KC = ET_24/ETREF_24, clamp(0,2.5)

Tpot = (1-exp(-0.6×LAI))×ETpot                            [k_ext=0.6]

Tact = (ET_24/ETpot, clamp 0-1) × Tpot

Eact = ET_24 - Tact

### soil_moisture.py

SMAP L4 — eng yaqin (±3 soat) o'lchov, o'rtacha EMAS

sm_surface, sm_rootzone, sm_rootzone_wetness — to'g'ridan-to'g'ri SMAP'dan

stress = wetness/0.4, clamp(0,1)                          [FAO-56 p=0.4]

### biomass.py

FPAR = 1.257×NDVI - 0.161                                 [Bastiaanssen & Ali 2003]

Rs24 (Rn24'dan tiklanadi) = (Rn24+110×τsw)/(1-α)

PAR = 0.48×Rs24

APAR = FPAR×PAR

heat_stress = 1-((Ta-25)/(25-5))², clamp(0.05,1.0)          [T_opt=25°C, T_cold=5°C]

vapor_stress = 1-0.2×VPD, clamp(0.05,1.0)                   [k_vpd=0.2]

LUE = 2.5 × heat_stress × vapor_stress × moisture_stress    [LUEmax=2.5 gC/MJ]

Biomass = APAR×0.0864×LUE × 10 × 2.0                        [kg DM/ha/day]

### irrigation.py

Sug'orish klassi (moisture_stress bo'yicha):

>0.85 → 0 (kerak emas) | 0.65-0.85 → 1 | 0.45-0.65 → 2 | <0.45 → 3

NDVI<0.15 → 0 (ekin yo'q)

Sug'orish chuqurligi (hardcoded): sm_depth = max(0.5-wetness, 0)×300.0 [mm]

Eslatma: compute_rn24() (radiation.py) formula sifatida to'g'ri yozilgan, lekin hech qayerda chaqirilmaydi (avvalgi tekshiruvda tasdiqlangan) — daily_et.py xuddi shu formulani mustaqil qayta yozadi. Chiziqli sxemangizga qo'shsangiz, buni "parallel, duplikat kod" deb belgilang, ikkita alohida qadam emas.