QADAM 23–26 — process_tile()dan qaytish → Kc monthly ET → AW → Drive CSV

Endi aynan current run_sebal.py Drive’ga yetadigan real yo‘lga keldik.

Current konfiguratsiya:

mode            = SEBAL_Milliy_Kc

process_by_tile = True

tile            = P30_R36

export_csv      = True

export_daily    = False

export_monthly  = False

save_cuirr      = False

save_aw         = True

csv_scale       = 30 m

folder          = SEBAL_AW_Bushland_2021

QADAM 23 — process_tile() qaytdi

Oldingi qadam oxirida:

return scene_images, info

Main’da:

scenes, info = process_tile(...)

bo‘ladi.

Types:

scenes → Python list[ee.Image]

info   → Python dict

Current scenes ichida faqat:

cloud filterdan o'tgan

+

anchor valid

+

energy balance tugagan

+

daily ET tugagan

scene’lar bor.

23.1. scenes bo‘sh bo‘lsa

Kod:

if not scenes:

continue

Demak:

CSV ❌

AW ❌

Drive task ❌

shu tile tugaydi.

23.2. scenes mavjud — current intended path

Keyin:

if export_csv and csv_region is not None:

Current:

True AND True

demak:

_export_zonal_csv(...)

ishga tushadi.

23.3. Daily raster branch

if export_daily:

Current:

False

Shuning uchun:

Export.image.toDrive() daily GeoTIFF ❌

23.4. Monthly raster branch

if export_monthly:

Current:

False

Shuning uchun:

monthly GeoTIFF ❌

va eng muhimi:

dr_carry

oydan oyga DR_END uzatish

ham current run’da ishlamaydi.

Current Drive yo‘li faqat:

table CSV export\boxed{\text{table CSV export}}

QADAM 23A — Kc mode sabab scene CSV’lar SKIP

_export_zonal_csv():

if cfg.is_kc_mode(mode):

SCENE_GROUPS = {}

Outer mode hali:

SEBAL_Milliy_Kc

Shuning uchun:

cfg.iskcmode=Truecfg.is_kc_mode=True

Demak current run’da:

INST CSV             ❌

DAILY_ET CSV         ❌

INST_KOMPONENT CSV   ❌

LST diagnostic CSV   ❌

Hammasi skip.

Sabab kod commentida:

Kc scene’lari SEBAL_Milliy scene processing bilan aynan bir xil; Kc mode’ning noyob mahsuloti monthly natija.

23A.1. LST diagnostic

if not cfg.is_kc_mode(mode):

Current:

False

Demak:

LSTDIAG ❌

Oldingi SEBAL_B validation trace’dan katta farq shu.

QADAM 23B — Monthly CSV bandlari

Boshlanishida:

mon_bands = ['ET_MONTHLY']

Current:

save_cuirr=False

shuning uchun:

PRZ

CUIRR

NIWR

AW_CU

ETPOT_MONTHLY

RUNOFF_MONTHLY

DEEPPERC_MONTHLY

qo‘shilmaydi.

Lekin:

save_aw=True

Shuning uchun:

mon_bands += [

'AW',

'AW_Eff',

'AVAILABLE_WATER',

'DP_MONTHLY',

'N_IRRIG',

'TAW'

]

Final current:

ET_MONTHLY

AW

AW_Eff

AVAILABLE_WATER

DP_MONTHLY

N_IRRIG

TAW

QADAM 24 — Qaysi oylar hisoblanadi?

Kod:

scene_months = sorted({

d[:7] for d in info.get('dates', [])

})

Masalan:

2021-04

2021-05

...

Muhim audit

info['dates']:

preprocessing collection sanalari.

Bu anchor valid bo‘lgandan keyingi scene_images sanalari emas.

Shuning uchun:

bir oyda Landsat input bor

lekin o'sha oyning barcha anchorlari fail

bo‘lsa ham o‘sha oy scene_months ichida qolishi mumkin.

Monthly Kc esa surviving season scenesdan NDVI interpolate qilib hisoblashga urinadi.

QADAM 24A — compute_monthly_et()

Har oy:

monthly = daily_et.compute_monthly_et(

scenes,

roi,

year,

month,

mode='SEBAL_Milliy_Kc',

utc_offset=-6

)

Muhim:

Bu yerga outer original mode kiradi:

SEBAL_Milliy_Kc

scene ichidagi SEBAL_Milliy emas.

Shuning uchun:

if cfg.is_kc_mode(mode):

TRUE.

Va:

ndvi_kc.compute_monthly_et_kc(...)

ishga tushadi.

QADAM 24B — Kc monthly ET energy balance’dan olinmaydi

Current final oylik ET:

EVAP_FRAC ❌

SOLAR_FRAC ❌

LAMBDA_E ❌

H ❌

dan qurilmaydi.

Asosiy formula:

ETd=(Kcb+Ke)ETo\boxed{ ET_d=(K_{cb}+K_e)ETo }

va:

ETMONTHLY=∑dETd\boxed{ ET_{MONTHLY}=\sum_d ET_d }

Scene energy balance faqat NDVI scene’larini yetkazib berish uchun ham mavjud.

QADAM 24C — Daily NDVI

ndvi_coll = ee.ImageCollection(image_list).select('NDVI')

Har kun uchun _ndvi_interp().

Ikki scene orasida:

w=t−tbta−tbw= \frac{t-t_b}{t_a-t_b} NDVId=(1−w)NDVIb+wNDVIa\boxed{ NDVI_d= (1-w)NDVI_b+wNDVI_a }

Bu haqiqiy temporal linear interpolation.

Oraliqdan tashqarida

Birinchi scene’dan oldin:

first scene NDVI

ushlab turiladi.

Oxirgisidan keyin:

last scene NDVI

ushlab turiladi.

QADAM 24D — Current crop branch

QADAM 2 da:

cfg.CROP_ASSETS=None

Shuning uchun:

PER_CROP=False

Current universal Bushland/paxta-kalibrlangan branch.

Agar crop_assets berilsa

PER_CROP=True

va har pixel uchun crop code’dan:

kcb_max

kcb_end_frac

sen_len

senescence on/off

root depth

p fraction

olinadi.

Kod 0 bo‘lgan non-crop pixel masklanadi.

Currentda bu yo‘q.

QADAM 24E — Current Kcb

Config:

a       = 1.25

b       = -0.10

kcb_max = 1.00

Formula:

Kcb=clamp(1.25NDVI−0.10, 0, 1.0)\boxed{ K_{cb} = clamp(1.25NDVI-0.10,\ 0,\ 1.0) }

24E.1. Senescence

Current:

senescence=True

sen_len=60 days

kcb_end_frac=0.45

Avval har pixelda:

eng yuqori NDVI bo‘lgan scene vaqti

qualityMosaic('NDVI') bilan olinadi.

Cho‘qqidan keyin:

days_after≥0days\_after\ge0

va Kcb asta-sekin pasayadi.

60 kun oxirida factor:

0.45\boxed{0.45}

gacha tushadi.

QADAM 24F — Canopy fraction

Config:

NDVIbare=0.15NDVI_{bare}=0.15 NDVIfull=0.85NDVI_{full}=0.85

Formula:

fc=clamp(NDVI−0.150.85−0.15,0,1)\boxed{ f_c= clamp \left( \frac{NDVI-0.15}{0.85-0.15}, 0,1 \right) }

Ochiq tuproq:

few=1−fc\boxed{f_{ew}=1-f_c}

QADAM 24G — Topsoil TEW/REW

Dataset:

SoilGrids 2.0

sand_mean

clay_mean

0–5 cm va 5–15 cm o‘rtachalanadi.

Saxton-Rawls:

sand,clay→FC,WPsand,clay\rightarrow FC,WP

TEW

TEW=1000(FC−0.5WP)×0.10\boxed{ TEW= 1000(FC-0.5WP)\times0.10 }

Minimum:

TEW≥1mmTEW\ge1mm

REW

REW=clamp(0.15Clay+2, 2, 11)\boxed{ REW= clamp(0.15Clay+2,\ 2,\ 11) }

va:

REW≤TEWREW\le TEW

QADAM 24H — Monthly Kc modelning boshlang‘ich De

init = TEW.rename('De')

Demak:

De0=TEW\boxed{De_0=TEW}

Ya’ni har oy boshida topsoil evaporation layer to‘liq quruq.

Oylar orasida De uzatilmaydi.

24H.1. Har kun CHIRPS

P = CHIRPS precipitation

Agar image bo‘lmasa:

P=0P=0

24H.2. Topsoil wetting

De2=max⁡(De−P,0)\boxed{ De_2=\max(De-P,0) }

Bu Kc modelning topsoil balansida:

runoff ❌

deep percolation ❌

alohida hisoblanmaydi.

To‘liq P topsoil wettingga qo‘llanadi.

QADAM 24I — Kr

Agar:

De2≤REWDe_2\le REW Kr=1Kr=1

Aks holda:

Kr=TEW−De2TEW−REW\boxed{ Kr= \frac{TEW-De_2}{TEW-REW} }

va:

0≤Kr≤10\le Kr\le1

QADAM 24J — Ke

Current:

Kcmax=1.15Kc_{max}=1.15 ke_scale=0.30ke\_scale=0.30

Formula:

Ke=min⁡[Kr(Kcmax−Kcb),fewKcmax]×0.30\boxed{ Ke= \min[ Kr(Kc_{max}-Kcb), f_{ew}Kc_{max} ] \times0.30 }

va pastdan 0.

QADAM 24K — ETo

Current config:

MILLIY_KC['ref_type']='grass'

Demak:

ETo=grass reference ET\boxed{ETo=\text{grass reference ET}}

Current source:

era5

va UTC−6 local calendar day.

24K.1. Daily ET

Transpiration:

T=Kcb×ETo\boxed{T=Kcb\times ETo}

Evaporation:

E=Ke×ETo\boxed{E=Ke\times ETo}

Final:

ETa=T+E\boxed{ET_a=T+E}

24K.2. De update

Denew=min⁡(De2+E, TEW)\boxed{ De_{new}= \min(De_2+E,\ TEW) }

Faqat soil evaporation E topsoil depletionni yana oshiradi.

24K.3. Monthly sum

ETMONTHLY=∑(T+E)\boxed{ ET_{MONTHLY} = \sum(T+E) }

Current PER_CROP=False, shuning uchun crop mask bilan final mask qilinmaydi.

QADAM 25 — save_aw=True: root_zone_water.compute_awnet()

Monthly ET yaratilgach:

if save_aw:

Current:

True\boxed{True}

Shuning uchun:

awimg = root_zone_water.compute_awnet(

scenes,

roi,

yr,

mo,

utc_offset=-6

)

chaqiriladi.

Manba: root_zone_water.py

25.1. Juda muhim: bu boshqa suv balansi

Kc monthly modelda:

De

— faqat yuqori 10 cm soil evaporation layer.

AW modelda esa yana:

Dr

— root-zone depletion.

Demak:

De ≠ Dr

QADAM 25A — Root-zone TAW

soil_water_params():

TAW=1000(FC−WP)Zr\boxed{ TAW= 1000(FC-WP)Z_r }

Current:

crop_assets=None

shuning uchun:

Zr=1.0mZ_r=1.0m

config default.

Demak:

TAW=1000(FC−WP)×1.0\boxed{ TAW=1000(FC-WP)\times1.0 }

per-pixel raster.

Per-crop TRUE branch

Crop asset bo‘lsa:

ZrZ_r

har crop uchun alohida crop_kc_tabledan olinadi.

Currentda yo‘q.

QADAM 25B — RAW

Current crop asset yo‘q:

p=0.5p=0.5

Shuning uchun:

RAW=0.5TAW\boxed{ RAW=0.5TAW }

Bu sug‘orish trigger chegarasi.

QADAM 25C — SCS Curve Number

Soil texture:

Sand > 50%

CN=67CN=67

Clay > 40%

CN=85CN=85

Aks holda

CN=78CN=78

Retention

S=25400CN−254\boxed{ S=\frac{25400}{CN}-254 }

minimum 1 mm.

Initial abstraction:

Ia=0.2S\boxed{I_a=0.2S}

QADAM 25D — Root-zone AW ham Kcb+Ke ETni qayta hisoblaydi

root_zone_water monthly ET_MONTHLYni tayyor monthly image’dan olmaydi.

U har kun qaytadan:

NDVI interpolation

Kcb

senescence

Ke

grass ETo

CHIRPS

hisoblaydi.

Demak AW calculation:

ETad=(Kcb+Ke)EToETa_d=(Kcb+Ke)ETo

bilan o‘z ichida yana bir kunlik model yuritadi.

25D.1. Topsoil De

AW model ichida ham:

De0=TEWDe_0=TEW

ya’ni har oy topsoil quruq boshlanadi.

QADAM 25E — Eng muhim current bug/audit: Dr boshlang‘ich holati

compute_awnet() signature:

def compute_awnet(

...,

dr_init_frac=0.0,

dr_init_img=None,

start_at_raw=False

)

Current caller:

compute_awnet(

scenes, roi, yr, mo,

utc_offset=...

)

ya’ni dr_init_frac uzatilmaydi.

Shuning uchun:

dr_init_frac=0.0\boxed{dr\_init\_frac=0.0}

Kod branchi

if dr_init_img is not None:

...

elif dr_init_frac is not None:

Dr0 = TAW * dr_init_frac

else:

Dr0 = RAW

Current:

dr_init_img = None

dr_init_frac = 0.0 ≠ None

demak:

Dr0=0\boxed{Dr_0=0}

25E.1. Bu nimani anglatadi?

Oy boshida root zone:

Dr=0Dr=0

ya’ni:

tuproq field capacityda\boxed{\text{tuproq field capacityda}}

deb boshlanadi.

25E.2. Docstring bilan nomuvofiqlik

Docstring:

DEFAULT — Dr = RAW

degan ma’noni beradi.

Lekin active signature:

dr_init_frac=0.0

sabab default real execution:

Dr=0, RAW emas\boxed{Dr=0,\ RAW\ emas}

Bu juda muhim kod bug/documentation mismatch.

25E.3. start_at_raw ishlatiladimi?

Signature’da:

start_at_raw=False

bor.

Lekin body’da bu variable biror marta tekshirilmaydi.

Ya’ni:

start_at_raw=True

bersangiz ham active code’ga ta’siri yo‘q.

Bu ham real audit.

QADAM 25F — Oylararo Dr currentda uzatiladimi?

Yo‘q.

CSV branch:

compute_awnet(...)

ga:

dr_init_img

bermaydi.

Shuning uchun har oy:

Dr0=0\boxed{Dr_0=0}

dan mustaqil boshlanadi.

Agar export_monthly=True raster branch ishlasa

main.pyda:

dr_carry

mexanizmi bor.

Ammo current:

export_monthly=False

Shuning uchun u yo‘lga umuman kirmadik.

QADAM 25G — Har kun root-zone runoff

CHIRPS:

PP

Curve Number runoff:

RO=(P−Ia)2P+0.8S\boxed{ RO= \frac{(P-I_a)^2} {P+0.8S} }

lekin:

P−IaP-I_a

pastdan 0 ga clamp qilinadi.

Demak kichik yog‘inda:

RO=0RO=0

QADAM 25H — Root-zone balance

Current:

CR=0CR=0

kapillyar ko‘tarilish yo‘q.

Formula:

x=Dr+ETa−(P−RO)−CR\boxed{ x= Dr+ETa-(P-RO)-CR }

Current:

CR=0CR=0

Agar x<0x<0

Suv field capacity’dan ortiq:

DP=−x\boxed{DP=-x} Drbefore=0Dr_{before}=0

Agar x≥0x\ge0

DP=0DP=0 Drbefore=xDr_{before}=x

QADAM 25I — Sug‘orish trigger

Condition:

Drbefore≥RAW\boxed{ Dr_{before}\ge RAW }

FALSE

Sug‘orish yo‘q:

Inet=0Inet=0 Drend=DrbeforeDr_{end}=Dr_{before}

TRUE

Sug‘orish yoqiladi:

Inet=Drbefore\boxed{ Inet=Dr_{before} }

va:

Drend=0\boxed{ Dr_{end}=0 }

Ya’ni model root zonani to‘liq field capacitygacha qayta to‘ldiradi.

25I.1. Inet nimani anglatadi?

Bu:

shu kun root zonani field capacitygacha yetkazish uchun zarur net irrigation.

Har oy:

AW=∑Inet\boxed{ AW=\sum Inet }

QADAM 25J — N_IRRIG

Har trigger kunida:

fire = 1

Aks holda:

0

Oylik:

N_IRRIG=∑fire\boxed{ N\_IRRIG=\sum fire }

Bu per-pixel sug‘orish hodisalari soni.

QADAM 25K — AW_Eff

Config:

efficiency=0.55efficiency=0.55

Current:

AW_Eff=AW0.55\boxed{ AW\_Eff= \frac{AW}{0.55} }

Bu gross/applied amount.

Masalan:

AW=100mmAW=100mm

bo‘lsa:

AW_Eff≈181.8mmAW\_Eff\approx181.8mm

faqat tushunish uchun.

Namingga ehtiyot

Current code’da:

AW      = net irrigation

AW_Eff  = AW / efficiency = gross applied water

AW_Eff nomi “efficiency-adjusted gross” degani.

QADAM 25L — AVAILABLE_WATER

Oy oxirida:

AVAILABLE_WATER=TAW−Drend\boxed{ AVAILABLE\_WATER = TAW-Dr_{end} }

Minimum 0.

Bu:

root zonada oy oxirida mavjud o‘simlikka foydalanish mumkin bo‘lgan suv.

25L.1. DP_MONTHLY

DP_MONTHLY=∑DP\boxed{ DP\_MONTHLY=\sum DP }

Deep percolation.

25L.2. TAW

Root-zone total available water capacity raster.

25L.3. DR_END

compute_awnet() aslida:

DR_END

band ham yaratadi.

Lekin _export_zonal_csv():

awimg.select([

'AW',

'AW_Eff',

'AVAILABLE_WATER',

'DP_MONTHLY',

'N_IRRIG',

'TAW'

])

qiladi.

Demak:

DR_END ❌ CSVga qo‘shilmaydi

va keyingi oyga ham uzatilmaydi.

QADAM 25M — Current monthly image final

Avval:

monthly:

ET_MONTHLY

Keyin AW image’dan:

AW

AW_Eff

AVAILABLE_WATER

DP_MONTHLY

N_IRRIG

TAW

qo‘shiladi.

Final:

ET_MONTHLY

AW

AW_Eff

AVAILABLE_WATER

DP_MONTHLY

N_IRRIG

TAW

QADAM 26 — Parcel bo‘yicha mean + median

Current csv_region:

NE

SE

NW

SW

Bushland parcel geometries.

Reducer:

ee.Reducer.mean().combine(

ee.Reducer.median(),

sharedInputs=True

)

Har bir monthly raster uchun 30 m scale’da:

reduceRegions(

collection=region_fc,

reducer=reducer,

scale=30

)

Har parcel uchun

Masalan:

ET_MONTHLY_mean

ET_MONTHLY_median

AW_mean

AW_median

AW_Eff_mean

AW_Eff_median

AVAILABLE_WATER_mean

AVAILABLE_WATER_median

DP_MONTHLY_mean

DP_MONTHLY_median

N_IRRIG_mean

N_IRRIG_median

TAW_mean

TAW_median

va:

year

month

name

properties.

26.1. N_IRRIG_mean nega decimal bo‘lishi mumkin?

Har pixelda:

N_IRRIG

integer count.

Lekin parcel bo‘yicha mean:

2.372.37

kabi decimal chiqishi mumkin.

Bu:

parcel ichidagi piksel sug‘orish sonlarining o‘rtachasi.

Median esa masalan 2 yoki 3.

26.2. Barcha oylar bitta FeatureCollection

Har oy:

month_fcs.append(...)

Oxirida:

month_fc = ee.FeatureCollection(month_fcs).flatten()

Demak:

oy 1 × 4 parcel

oy 2 × 4 parcel

...

hammasi bitta CSVga yig‘iladi.

QADAM 26A — Export.table.toDrive()

Endi nihoyat Drive task yaratiladi:

t2 = ee.batch.Export.table.toDrive(

collection=month_fc,

description=f'{folder}_MONTHLY_ET{prefix}',

folder=folder,

fileNamePrefix=f'{folder}_MONTHLY_ET{prefix}',

fileFormat='CSV'

)

Current:

folder =

SEBAL_AW_Bushland_2021

Tile:

P30_R36

Prefix:

_P30_R36

26A.1. Current task/file nomi

Conceptually:

SEBAL_AW_Bushland_2021_MONTHLY_ET_P30_R36

Drive folder:

SEBAL_AW_Bushland_2021/

CSV:

SEBAL_AW_Bushland_2021_MONTHLY_ET_P30_R36.csv

QADAM 26B — .start()

Keyingi eng muhim qator:

t2.start()

Mana shu nuqtada:

Earth Engine batch task queue’ga yuboriladi\boxed{\text{Earth Engine batch task queue’ga yuboriladi}}

toDrive() nima qiladi?

Task object yaratadi.

Hali hisoblashni boshlamaydi.

.start() nima qiladi?

Taskni GEE backendga submit qiladi:

READY

↓

RUNNING

↓

COMPLETED yoki FAILED

26B.1. Drive shu zahoti fayl oladimi?

Yo‘q.

.start() paytida:

Drive file hali mavjud bo‘lishi shart emas

GEE backend:

monthly Kc graph

+

AW water balance

+

reduceRegions

+

CSV serialization

ni bajarib tugatgandan keyin Drive’da fayl paydo bo‘ladi.

26B.2. Task tracking

tasks.append(t2)

Demak current Kc tile uchun _export_zonal_csv():

1 ta tracked task\boxed{1\ ta\ tracked\ task}

qaytaradi.

Scene CSV/LST diag skip qilinganligi sabab boshqa table task yo‘q.

CURRENT run uchun Drive’gacha to‘liq oqim

run_sebal.py

│

▼

SEBAL_Milliy_Kc

│

▼

process_tile()

│

▼

valid scene_images

│

▼

_export_zonal_csv()

│

├─ Kc mode?

│     YES

│

├─ INST CSV ❌

├─ DAILY CSV ❌

├─ COMPONENT CSV ❌

└─ LSTDIAG ❌

│

▼

scene_months

│

har oy uchun

│

▼

compute_monthly_et()

│

▼

is_kc_mode=True

│

▼

NDVI linear interpolation

│

Kcb + Ke

│

grass ETo

│

▼

ET_MONTHLY

│

save_aw=True

│

▼

root_zone_water

│

├─ TAW

├─ RAW=0.5TAW

├─ CHIRPS

├─ SCS-CN runoff

├─ Dr balance

├─ irrigation trigger

└─ ΣInet

│

▼

AW / AW_Eff / Available / DP / N_IRRIG / TAW

│

▼

reduceRegions()

4 Bushland parcels

mean + median

│

▼

FeatureCollection all months

│

▼

Export.table.toDrive()

│

▼

.start()

│

▼

GEE Batch Task

│

▼

Google Drive CSV

QADAM 23–26 dagi eng muhim topilmalar

- Current export_daily=False → GeoTIFF daily export yo‘q.

- Current export_monthly=False → monthly GeoTIFF yo‘q.

- export_csv=True → real Drive yo‘li _export_zonal_csv().

- Kc mode sabab scene-level 4 CSV turlari ham skip.

- Current Kc run’da faqat MONTHLY_ET CSV task yaratiladi.

- Final monthly ET energy balance’dan emas:

ET=(Kcb+Ke)EToET=(Kcb+Ke)ETo

- Current crop_assets=None → universal paxta/Bushland Kcb.

- Current:

Kcb=clamp(1.25NDVI−0.10,0,1)Kcb=clamp(1.25NDVI-0.10,0,1)

- NDVI haqiqiy linear interpolatsiya.

- Kc topsoil model har oy:

De0=TEWDe_0=TEW

bilan boshlanadi.

- Topsoil Kc balansida yog‘inning hammasi Deni kamaytiradi; runoff u yerda yo‘q.

- save_aw=True alohida root-zone water balance’ni ishga tushiradi.

- Root-zone:

TAW=1000(FC−WP)ZrTAW=1000(FC-WP)Z_r

- Current crop_assets=None → Zr=1mZ_r=1m.

- Current:

RAW=0.5TAWRAW=0.5TAW

- AW balansida SCS Curve Number runoff bor.

- Sug‘orish:

Dr≥RAWDr\ge RAW

bo‘lganda ishga tushadi va root zone FCgacha to‘ldiriladi.

- AW = ΣInet — net irrigation.

- AW_Eff = AW/0.55 — gross applied water.

- Juda muhim bug: compute_awnet() docstring “default RAW” desa ham active default:

dr_init_frac=0⇒Dr0=0dr\_init\_frac=0 \Rightarrow Dr_0=0

ya’ni har oy field capacitydan boshlanadi.

- start_at_raw parameter mavjud, lekin active body’da ishlatilmaydi.

- Current CSV branchda DR_END keyingi oyga uzatilmaydi.

- Har oy AW mustaqil hisoblanadi.

- Parcel statistikasi 30 m’da mean + median.

- Export.table.toDrive() task yaratadi.

- .start() — Drive exportning real submit nuqtasi.

Kod manbalari: main.py, daily_et.py, ndvi_kc.py, root_zone_water.py, consumptive_use.py.

Keyingi QADAM 27 da .start()dan keyin qolgan oxirgi zanjirni yopamiz: tasks → all_tasks → main.run() return, GEE task READY/RUNNING/COMPLETED/FAILED, Drive’da aynan nima paydo bo‘ladi va current run_sebal.py taskni kutadimi yoki dastur darhol tugaydimi.